BACSE324 · Embedded Systems Architecture · Fall 2026 · Review 2

# Design Models: CDFG, FSM and Task Schedule

Review 2 submission · due end of September · 6–10 pages excluding cover and appendices

| Project title | Gesture-Controlled Smart Glove for Human–Computer Interaction |
| --- | --- |
| Scope tier (from review sheet) | Tier 2 — full embedded system |
| Submission date | September 2026 |
| Repository link (optional) | github.com/dayaj1222/data-glove |

Team

| Member | Reg. no. | Module owned | Sections written |
| --- | --- | --- | --- |
| Dayakrishn Joshi | 25BCE2951 | ESP-32 firmware, MPU-6500 I²C + flex ADC acquisition | §1, §2, §4 |
| Syed Mohammad Ayaan | 25BCE2311 | ESP-32 wireless link, PC recognition pipeline | §3, §5, §6, §7 |

# 1  System recap

The Smart Glove is a wearable input device that converts hand pose (two flex sensors) and hand motion (an MPU-6500 6-axis IMU) into standard keyboard and mouse events on a PC, for a single operator who wants hands-free control. The ESP-32 firmware samples both flex ADC channels and the IMU over I²C at a fixed rate and frames them into a fixed-width binary record: five 16-bit flex counts followed by nine 32-bit floats (accelerometer x, y, z, gyroscope x, y, z, magnetometer x, y, z; the magnetometer slots always read zero because the MPU-6500 has none) — a 46-byte datagram, parsed on the host by glove/protocol/simple.py::SimpleBinaryProtocol.parse using the <5H9f struct format.

Frames arrive over UDP (default port 5005) through glove/transport/udp.py::UDPTransport.recv, are pushed through glove/pipeline/main.py::Pipeline.run, classified, and dispatched to an action binding. On the PC a portable recognition layer classifies the gesture and an OS-specific layer injects it as input (uinput on Linux, SendInput on Windows). The host pipeline is transport- and recogniser-agnostic — Transport, Protocol, Recognizer and Action are all interfaces — so the same code runs against the mock transport used in unit tests and the live UDP link.

Deviations from the approved proposal. Two changes were made at the design stage: (i) the proposed MPU6050 was replaced by the MPU-6500 (6-axis accelerometer + gyroscope, I²C, WHO_AM_I = 0x70) for availability. The part has no magnetometer, so there is no absolute heading: pitch and roll come from the accelerometer, and yaw is available only as an integrated gyro rate; (ii) only two flex sensors are fitted rather than five, since flex sensors are the most expensive part and two (index + thumb) are sufficient to demonstrate distinct poses in a proof-of-concept. The wire format already carries five flex slots, so the two fitted sensors populate flex[0] and flex[1] and the remaining slots read zero; no host change is needed when the other three are added.

Figure 1 — System block diagram: glove sensors → ESP-32 → PC software stack → applications.

## 1.1  Conditions carried from proposal review

| Condition / mandatory addition (from your review sheet) | Status | Where it is addressed |
| --- | --- | --- |
| Justify the critical function against ≥3 candidates | Done | §2.1 |
| Provide a CDFG with node costs and the critical path | Done | §2.2–2.4 |
| Provide a system FSM with a fault and safe state | Done | §3 |
| State the WCET source and a schedulability proof | Done | §4.1, §4.3 |

# 2  Control Data Flow Graph

## 2.1  Choosing the critical function

| Function / ISR | Trigger | Rate | Est. execution time | Deadline | Chosen? Why / why not |
| --- | --- | --- | --- | --- | --- |
| Window inference (features + model.predict) | prediction timer | 4 Hz | ~15 ms | 250 ms | Chosen. Longest execution, most data dependencies, blocks action dispatch. |
| Frame poll (wireless RX) | poll | 30 Hz | ~2–5 ms | 33 ms | No — small, stateless. |
| Frame parse (SimpleBinaryProtocol) | per datagram | 30 Hz | <0.5 ms | 33 ms | No — trivial fixed cost. |

Justification: Window inference in backend/app/routers/training.py::predict has the longest execution time, consumes the whole buffered window (most data dependencies), and blocks the action dispatch that follows it.

## 2.2  The CDFG

Figure 2 — CDFG of predict(). Circles = operations, diamonds = decisions, solid = data edges, dashed = control edges, grouped into basic blocks BB0–BB3.

## 2.3  Node table

| Node | Operation | Inputs | Output | Est. cost (cycles / µs) | Source line or function |
| --- | --- | --- | --- | --- | --- |
| N1 | Check window ≥10 frames | frame buffer | drop | 200 | predict |
| N2 | Split into 4 sensor blocks | frames | blocks | 400 | extract_features |
| N3 | Per-axis mean/std/min/max | block | 4 stats/axis | 2500 | extract_features |
| N4 | Concat into 64 features | stats | vector | 300 | extract_features |
| N5 | Shape check vs model | features, model | ok / HTTP 400 | 100 | predict |
| N6 | Load pickled model | path | model | 1500 | predict |
| N7 | Reshape (1, −1) | features | 2-D array | 50 | predict |
| N8 | predict + predict_proba | features, model | id, conf | 8000 | predict |
| N9 | Confidence ≥ threshold? | conf, thr | bool | 50 | predict_and_act |
| N10 | Look up action binding | gesture id | record | 300 | predict_and_act |
| N11 | Execute action (uinput) | config | input event | 2000 | actions/* |

All costs are estimated (Python-level, dev PC); none measured yet.

## 2.4  Critical path

| Critical path (node sequence) | Total latency | Deadline | Slack |
| --- | --- | --- | --- |
| window → split → stats → concat → load → predict → conf → lookup → execute | ~15.3 ms | 250 ms | ~234.7 ms |

What the CDFG tells us about the implementation: The model load (1.5 ms) is on the path every call because predict re-opens the .pkl. This dependency will be broken: the model is cached at startup in the Final build. Recomputing full-window statistics (2.5 ms) could be made incremental, but slack is ample, so it is deferred.

# 3  System-behaviour FSM

## 3.1  Model type

Model type: Mealy. Outputs depend on state and input — a recognised gesture emits an action only when confidence ≥ threshold. A Moore machine cannot express “act only if confident”.

## 3.2  State diagram

Figure 3 — System FSM. Every transition is labelled event [guard] / action.

## 3.3  State transition table

| Current state | Event | Guard | Next state | Output / action |
| --- | --- | --- | --- | --- |
| IDLE | start | sensors available | STREAMING | open transport, buffer |
| IDLE | start | sensors unavailable | FAULT | sensor-unavailable error |
| STREAMING | frame | buffer < window | STREAMING | append frame |
| STREAMING | window full | bridge up, model present | TRACKING | run inference |
| STREAMING | link lost | — | FAULT | glove-disconnected error |
| TRACKING | tick | confidence < threshold | TRACKING | none (display “—”) |
| TRACKING | tick | confidence ≥ threshold | ACTING | dispatch action |
| ACTING | action done | binding found | TRACKING | inject key/mouse event |
| ACTING | action done | no binding | TRACKING | none (logged) |
| FAULT | recover | link/sensor restored | RECOVER | re-arm, clear buffer |
| FAULT | stop | — | IDLE | close transport |
| RECOVER | re-armed | buffer primed | STREAMING | resume streaming |

## 3.4  Fault, recovery and safe state

Safe state and outputs: FAULT outputs no keyboard or mouse events — no key held, no button pressed, no relative motion; every injected event ends with a release and syn(), so a stuck modifier (e.g. held super) can never lock the desktop. FAULT is reachable from every other state by the fatal-error / link-loss event. Recovery: recover → RECOVER, which clears the frame buffer (stale frames must never be inferred against) and returns to STREAMING only once primed. A watchdog transition (STREAMING/TRACKING → FAULT on link lost) prevents a dropped wireless link leaving the system silently half-running.

## 3.5  Completeness check

| State ↓ / Event → | start | frame | window full / re-armed | bridge down / recover |
| --- | --- | --- | --- | --- |
| IDLE | STREAMING | — | — | — |
| STREAMING | — | STREAMING | TRACKING | FAULT |
| TRACKING | — | — | — | FAULT |
| ACTING | — | — | — | FAULT |
| FAULT | — | — | — | RECOVER |
| RECOVER | — | — | STREAMING | FAULT |

# 4  Task schedule

## 4.1  Task set

| Task | Trigger | Period T | WCET C | Deadline D | Priority | U = C / T | WCET source |
| --- | --- | --- | --- | --- | --- | --- | --- |
| IMU sample (imuTask) | timer, core 0 | 10 ms | 0.42 ms | 10 ms | High | 0.042 | estimated (I²C @ 400 kHz) |
| Flex sample (flexTask) | timer, core 0 | 10 ms | 0.18 ms | 10 ms | High | 0.018 | estimated (ADC 12-bit) |
| Frame build (packetTask) | queue recv | 10 ms | 0.21 ms | 10 ms | Medium | 0.021 | estimated |
| Wireless TX (txTask) | frame ready | 10 ms | 0.90 ms | 10 ms | Medium | 0.090 | estimated (UDP sendto) |
| Host frame poll | timer | 33 ms | ~5 ms | 33 ms | High | 0.152 | estimated |
| Host inference tick | timer | 250 ms | 15.3 ms | 250 ms | Med | 0.061 | estimated |
| Watchdog (wdtTask) | timer | 3000 ms | 0.05 ms | 3000 ms | Low | 0.00002 | estimated |

Total utilisation U =  ≈ 0.17 on the ESP-32 (four firmware tasks) and ≈ 0.21 on the host (poll + inference + watchdog). All WCETs are estimated from the fixed peripheral costs (I²C transfer, ADC conversion, UDP sendto) until measured with a GPIO toggle and logic analyser.

## 4.2  Scheduling approach

Approach chosen: Task level: Rate-Monotonic (RMS). Operation level: ASAP list scheduling.

Why this approach and not the obvious alternative: The task set is small, fixed-rate and preemption-cheap, so RMS static priorities are simple and provable. EDF would allow higher utilisation but needs dynamic sorting and response-time checks; at U ≈ 0.21 it buys nothing. A cyclic executive adds jitter without benefit. ASAP gives the shortest schedule for a mostly-serial CDFG chain.

## 4.3  Schedulability analysis

RMS utilisation bound U ≤ n(21/n − 1), n = 3 (poll, inference, watchdog):

bound = 3(21/3 − 1) = 0.7798,     U = 5/33 + 15.3/250 + 1/3000 = 0.2130 ≤ 0.7798

On the ESP-32 the four firmware tasks run under FreeRTOS with rate-monotonic priorities (periods 10/10/10/3000 ms, U_fw = 0.171, well under the n = 4 bound of 0.757); each completes well inside its 10 ms frame because the samplers are pure blocking I²C/ADC reads with no dynamic allocation. Both task sets are schedulable by the utilisation bound. Worst-case response for the host inference task: R ≈ 15.3 + 5 (one poll interference) = 20.3 ms ≪ 250 ms (slack ≈ 229.7 ms), so no response-time analysis is required.

Operation level — ASAP control-step chain: window → split → stats → concat → load → predict → conf → lookup → execute; all nodes have zero mobility except the model load, removable by caching.

## 4.4  Timeline

Figure 4 — Schedule over one hyperperiod (task level): poll @ 33 ms, inference @ 250 ms, watchdog @ 3000 ms; each inference bar sits well inside its frame.

## 4.5  Shared resources and interrupts

| Shared resource | Used by | Protection | Worst-case blocking |
| --- | --- | --- | --- |
| SQLite glove.db | record / train / predict | session per request | one transaction; no ISR shares it |
| Frame buffer | poll (write), inference (read) | single-threaded | none observed |
| Model .pkl | inference, retrain | load-per-request | ~1.5 ms (removed by caching) |
| uinput device | action dispatch | one UInput/action; syn() | serialised by OS input layer |
| I²C bus | imuTask / flexTask | one bus mutex | one transaction |
| Wireless RX ring buffer | firmware txTask / host poll | single-producer ring | one buffer copy |

# 5  Planned traceability

| Model element (CDFG node, FSM state, task) | Planned file | Planned function | Owner |
| --- | --- | --- | --- |
| Window guard | PredictUI.js | runPrediction | DJ |
| Feature build | training.py | extract_features | DJ |
| Inference | training.py | predict | DJ |
| Dispatch | training.py | predict_and_act | DJ |
| Input injection | input/keyboard.py, mouse.py | tap / click | DJ |
| FSM states | stateMachine.js (new) | — | SA |
| Frame poll task | backend/app/bridge.py | get_latest_grouped | SA |
| Inference tick task | PredictUI.js | setInterval(…, 250) | DJ |
| Watchdog task (host) | frontend/src/components/RecordingUI.js | checkBridge | SA |
| IMU sampling task | firmware/src/imu.c | imuTask() | DJ |
| Flex ADC task | firmware/src/flex.c | flexTask() | DJ |
| Frame build task | firmware/src/packet.c | packetTask() | DJ |
| Wireless TX task | firmware/src/net.c | txTask() | SA |
| Watchdog task (firmware) | firmware/src/wdt.c | wdtTask() | SA |

# 6  Plan to Final

| Remaining work | Owner | Target date | Risk and fallback |
| --- | --- | --- | --- |
| Cache model in memory | DJ | early Oct | Low; fallback = load per request |
| Extract FSM module | SA | early Oct | Medium; fallback = FSM stays implicit |
| Integrate ESP-32 + MPU-6500 + flex on the glove | DJ | mid Oct | High (hardware); fallback = IMU-only |
| FreeRTOS task skeleton (imuTask/flexTask/txTask) | DJ | mid Oct | Medium; fallback = single super-loop |
| Measure firmware WCETs (GPIO toggle + logic analyser) | DJ | mid Oct | Medium; fallback = keep estimates |
| Fault-state unit tests | SA | late Oct | Low |

# 7  Individual contribution

| Member | Model / section produced | Module for Final | Share % |
| --- | --- | --- | --- |
| Dayakrishn Joshi | System recap, CDFG, task set | ESP-32 firmware + ADC acquisition | 50 |
| Syed Mohammad Ayaan | System FSM, schedulability, traceability, plan | ESP-32 WiFi/UDP link + PC recognition pipeline | 50 |

# References

[1]  Espressif Systems, ESP32 Series Datasheet, 2024.

[2] InvenSense, MPU-6500 Six-Axis (Gyro + Accelerometer) MEMS MotionTracking Device Product Specification, rev. 1.3.

[3] InvenSense, MPU-6500 Register Map and Descriptions, rev. 2.1.

[4] C. L. Liu and J. W. Layland, “Scheduling Algorithms for Multiprogramming in a Hard-Real-Time Environment,” J. ACM, 20(1):46–61, 1973.

# Submission checklist

| Item | ✓ |
| --- | --- |
| Guidance blocks deleted; 6–10 pages excluding cover and appendices | ✓ |
| Critical function justified against at least three candidates | ✓ |
| CDFG with node costs and the critical path stated | ✓ |
| FSM with model type, transition table, fault and safe state | ✓ |
| Task set with WCET source, chosen approach and schedule-ability proof | ✓ |
| Every carried condition listed with its status | ✓ |
| Every member named against a model they can explain | ✓ |